Section-A

[S1]

What factors should be considered when selecting the CPU, RAM, and storage to ensure good performance and hardware compatibility?

Ans:

1. Central Processing Unit (CPU)

- Performance Factors:

  - Core & Thread Count: Running IDEs, compilers, browsers, and video calls simultaneously requires parallel processing. A CPU with at least 6 to 8 cores (12 to 16 threads) prevents system slowdowns during concurrent tasks.

  - Clock Speed & IPC: High single-core boost clock speeds ensure quick response times for code execution, browser tab responsiveness, and IDE tasks.

  - Integrated Graphics / Hardware Encoders: An integrated GPU with dedicated media engines (e.g., QuickSync, AV1/H.264 support) offloads video conferencing rendering and background blur from main processing cores.

- Compatibility Factors:

  - Socket & Chipset: The CPU must match the motherboard socket (e.g., Intel LGA1700/LGA1851 or AMD AM5) and supported motherboard chipset.

  - Thermal & Cooler Compatibility: The CPU cooler must support the processor’s Thermal Design Power (TDP) to prevent thermal throttling under sustained compilation workloads.

2. Random Access Memory (RAM)

- Performance Factors:

  - Capacity: Web browsers (with multiple open tabs), programming IDEs, and video calls are highly memory-intensive. 32 GB (or a minimum of 16 GB) is essential to prevent disk paging and stutter.

  - Dual-Channel Architecture: Memory should be installed in matched pairs (e.g., 2 × 16 GB instead of 1 × 32 GB) to double the memory bandwidth.

  - Speed & Latency: High-frequency memory (e.g., DDR5-5600/6000 or DDR4-3200) improves compilation speeds and overall system responsiveness.

- Compatibility Factors:

  - DDR Standard: The RAM generation (DDR4 vs. DDR5) must match the physical slots and memory controller of the motherboard and CPU (they are not interchangeable).

  - Motherboard QVL & Clearance: RAM should be verified on the motherboard’s Qualified Vendor List (QVL) for stability, and module height must clear the CPU cooler.

3. Storage

- Performance Factors:

  - Interface/Drive Type: An NVMe M.2 PCIe SSD (Gen 3 or Gen 4) is required. Traditional HDDs or standard SATA SSDs create severe bottlenecks during file indexing, project builds, and dependency installs.

  - Random Read/Write Speeds (IOPS) & DRAM Cache: High random 4K read/write speeds ensure fast project loading, indexing, and multitasking. Drives with a dedicated DRAM cache or HMB sustain performance under heavy workloads.

  - Capacity: A minimum of 512 GB to 1 TB is necessary to store operating systems, development toolchains, caches, and application data comfortably.

- Compatibility Factors:

  - M.2 Form Factor & Protocol: The motherboard must have an M.2 slot supporting the 2280 form factor and PCIe NVMe protocol (not SATA-only).

  - Thermal Dissipation: High-speed NVMe drives require adequate heatsinks or motherboard thermal pads to avoid performance throttling under sustained writes.

[S2]

Explain how the antivirus software affects CPU, RAM, and storage performance, leading to system slowdowns.

Ans:

Antivirus software runs background services to monitor files, processes and network activity. It uses CPU resources when it scans files and checks running programs. It also uses RAM for its services, databases and scanning processes. If the computer already has limited RAM, the extra usage can make other applications feel slower. Antivirus software also performs disk reads and writes when it scans files or updates its virus definitions. If a full scan is running, storage activity can become high and other applications may have to wait for disk access. Therefore, the slowdown can happen because CPU, RAM and storage resources are being used by the antivirus at the same time. I would check the antivirus scan schedule and system resource usage before deciding whether the antivirus itself is the main problem.

[S3]

Describe the steps you would follow to troubleshoot the problem if it is caused by driver incompatibility or operating system issues.

Ans:

A machine maintains stability under heavy use through the combined efforts of parallel compute and thermal regulation:

- Offloading Compute Work (GPU): The GPU handles matrix-heavy and visual calculations concurrently, preventing system slowdowns by leaving the CPU available for general computing logic.

- Thermal Dissipation (Cooling System): Sustained electrical draw produces high temperatures. Thermal paste bridges microscopic surface gaps to conduct heat into heatsinks, while fan airflow forces the heat out of the chassis.

- Mitigating Throttling and Crashes: If operating temperatures exceed rated limits, modern processors reduce their clock speeds (thermal throttling) to protect against physical degradation. Extreme heat can lead to software crashes and blue screens.

- Outcome: A properly paired GPU provides sustained compute capability, while adequate thermal dissipation preserves high clock rates and system uptime during long rendering sessions.

[S4]

Explain how a UPS and surge protector help protect computer hardware and prevent data loss during these situations.

Ans:

How a Surge Protector and UPS Protect Hardware and Data

- Surge Protector (Protects Hardware):

  - Function: Detects sudden high-voltage spikes (common when power cuts or returns) and safely diverts excess electricity into the ground.

  - Benefit: Prevents electrical components like the motherboard and power supply from burning out.

  - Limitation: Has no battery, so it cannot prevent data loss during a complete blackout.

- UPS / Uninterruptible Power Supply (Prevents Data Loss & Regulates Power):

  - Function: Switches instantly to battery backup during a power cut and uses Automatic Voltage Regulation (AVR) to smooth out voltage drops (brownouts).

  - Benefit: Gives users time to save open work and shut down cleanly, preventing file corruption and lost data.

Key Difference at a Glance

| Device | Main Protection | Prevents Data Loss? |
| --- | --- | --- |
| Surge Protector | Shields hardware against dangerous voltage spikes. | No (PC turns off instantly during cuts). |
| UPS | Provides battery backup and stabilizes fluctuating voltage. | Yes (Keeps PC on so work can be saved). |

[S5]

Explain how DNS and DHCP work together in this network, and identify which service is most likely misconfigured based on the symptoms described.

Ans:

DHCP automatically provides network settings such as an IP address, subnet mask, default gateway and DNS server address to a client. This allows a new computer to join the network without manually configuring every setting. DNS translates names into IP addresses. For example, when a user enters the name of an internal application server, DNS should return the server's IP address. The symptom of being able to reach the application server by IP address but not by name points towards a DNS problem. However, the devices sometimes failing to obtain a valid IP address points towards a DHCP problem as well. Therefore, I would check both services. I would first verify that DHCP is giving clients valid IP addresses and the correct DNS server address. Then I would test DNS using tools such as nslookup and check the internal DNS records. The name-resolution failure is most directly related to DNS, while the invalid-IP symptom suggests a separate DHCP configuration or availability issue.

[S6]

Describe the troubleshooting steps you would take to determine whether the firewall rules are blocking legitimate traffic, and how you would resolve the issue.

Ans:

Troubleshooting Blocked Legitimate Traffic

- Verify Connectivity and Port Reachability

  - Ping the destination IP to test basic layer-3 reachability.

  - Test the specific application port using Test-NetConnection -ComputerName <IP> -Port <Port> on Windows or nc -zv <IP> <Port> on Linux.

  - Verification: A timeout indicates traffic is being dropped along the path.

- Check Firewall Drop Logs

  - Inspect the firewall’s live traffic logs (Windows Defender Firewall log, iptables log, or appliance log such as Cisco ASA/Palo Alto).

  - Search for entries marked DROP or DENY containing the client’s source IP and the destination service port.

  - Verification: The log displays an explicit drop entry confirming the exact rule or Access Control List (ACL) blocking the packet.

- Trace the Packet Path

  - Use built-in diagnostic tools like packet-tracer on hardware firewalls to simulate the packet flow.

  - Verification: The tool identifies the exact inspection phase where the packet fails (e.g., Drop-reason: (acl-drop)).

Section – B

Task 1: Workstation Assembly

Ans:

Workstation Assembly and POST Verification Procedure

In an environment where physical components or specialized toolkits are simulated or unavailable, the standard procedure follows a phased assembly order to ensure hardware safety and successful Power-On Self-Test (POST) completion.

Phase 1: Core Component Installation (Pre-Chassis Setup)

- Safety & Bench Setup: Establish an ESD-safe surface (such as an anti-static mat or the motherboard packaging box) to assemble core parts prior to mounting inside the chassis.

- Processor Installation: Align the pin-1 alignment indicator on the CPU with the motherboard socket marker, seat the chip flat into the socket frame, and latch down the retention lever.

- Thermal Management: Apply thermal compound (if not pre-applied), mount the CPU cooler securely with even cross-pattern tension, and connect its power lead directly to the CPU_FAN header.

- Memory Installation: Seat the RAM modules into the vendor-recommended dual-channel slots (typically A2 and B2) until the side locking clips click shut.

- Primary Storage: Fasten the M.2 NVMe SSD into the primary M.2 slot using its mounting screw/latch, ensuring the thermal shield pad is in contact with the drive.

Phase 2: Chassis Integration & Internal Wiring

- Chassis & Power Supply Mounting: Mount the Power Supply Unit (PSU) into its dedicated bay with the intake fan facing the chassis vent. Verify brass standoff alignments inside the case, fit the I/O shield, and fasten the motherboard securely.

- Main Power Delivery: Firmly latch the primary 24-pin ATX power harness to the motherboard and the 8-pin (4+4) EPS power cable to the CPU power socket.

- Discrete GPU Installation: Seat the graphics card firmly into the top primary PCIe x16 slot until the retention latch snaps, screw the bracket to the chassis frame, and plug in the required PCIe power leads.

- Front-Panel and Data Connections: Connect the chassis front I/O headers (Power Switch, Reset Switch, drive LEDs) according to the motherboard pin-out diagram, along with front USB headers and HD Audio.

Phase 3: Final Pre-Flight and POST Validation

- Clearance Check: Inspect internal routing to ensure cables are tied back clear of all fan blades and verify no loose mounting screws remain on the board.

- Initial Boot & POST Check: Connect the external power cord, switch on the PSU, and depress the power switch. Confirm that fans spin, onboard diagnostic status LEDs cycle and clear, and video output is sent to the display.

- Firmware Hardware Inventory: Access the UEFI/BIOS utility (using Del or F2) to verify that the central processor, total installed RAM capacity, and primary storage drive are accurately recognized by the system.

Task 2: RAM Mismatch Troubleshooting

Ans:

Here is the final, ready-to-copy version written in clean, simple English for your assignment:

Part A: Cisco Packet Tracer LAN Topology (5 PCs + 1 Printer)

1. IP Addressing Scheme (Class C: 192.168.10.0/24, Subnet Mask: 255.255.255.0)

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | FastEthernet0 | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| PC1 | FastEthernet0 | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 |
| PC2 | FastEthernet0 | 192.168.10.13 | 255.255.255.0 | 192.168.10.1 |
| PC3 | FastEthernet0 | 192.168.10.14 | 255.255.255.0 | 192.168.10.1 |
| PC4 | FastEthernet0 | 192.168.10.15 | 255.255.255.0 | 192.168.10.1 |
| Printer0 | FastEthernet0 | 192.168.10.20 | 255.255.255.0 | 192.168.10.1 |

2. Setup and Ping Verification Steps:

- Connect all 5 PCs and the printer to a single Cisco 2960 switch using copper straight-through cables.

- Configure static IP addresses on all systems:

  - PCs: Open Desktop > IP Configuration and enter the assigned IP and subnet mask.

  - Printer: Open Config > FastEthernet0 and enter the static IP and subnet mask.

- Open the Command Prompt on PC0 and verify connection using the ping command:

DOS

ping 192.168.10.20

Verification: The screen displays successful replies (Reply from 192.168.10.20) with 0% packet loss.

Part B: Task 2 – RAM Mismatch Troubleshooting

- Check Motherboard Diagnostics: Turn on the workstation and observe the motherboard debug LEDs, beep codes, or POST error display to verify if the issue is memory-related.

- Examine BIOS/UEFI Settings: Enter BIOS setup (Del or F2) during boot and compare the detected memory size, frequency, and timings with the physically installed RAM.

- Verify Compatibility: Check the labels on each RAM stick to confirm they share the same generation (e.g., DDR4), operating voltage, and latency, and verify support using the motherboard's Qualified Vendor List (QVL).

- Isolate Individual Modules: Unplug the power cord, follow ESD anti-static safety precautions, and test one RAM stick at a time in slot A2 to identify whether a specific stick or slot is faulty.

- Reset Memory Profiles: Turn off aggressive overclocking profiles (XMP/EXPO/DOCP) in the BIOS so all installed sticks run at standard, matched JEDEC base speeds.

- Save and Final Verification: Save settings (F10) and restart the computer. Open Windows Task Manager (Performance tab) to verify that the full RAM capacity is detected and operating stably.

Task-3

LAN Topology Build

Ans:

To construct a Local Area Network (LAN) using a Class C subnet (192.168.1.0/24) connecting 5 PCs and 1 network printer via a switch, and verify connectivity using the Ping command.

1. IP Addressing Table

| Device | Device Type | Port | IP Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| PC0 | PC | FastEthernet0/1 | 192.168.1.10 | 255.255.255.0 |
| PC1 | PC | FastEthernet0/2 | 192.168.1.11 | 255.255.255.0 |
| PC2 | PC | FastEthernet0/3 | 192.168.1.12 | 255.255.255.0 |
| PC3 | PC | FastEthernet0/4 | 192.168.1.13 | 255.255.255.0 |
| PC4 | PC | FastEthernet0/5 | 192.168.1.14 | 255.255.255.0 |
| Printer0 | Printer | FastEthernet0/6 | 192.168.1.20 | 255.255.255.0 |

2. Steps to Configure in Packet Tracer

- Add Devices: Drag 1 Switch (Cisco 2960), 5 PCs (PC0 to PC4), and 1 Printer (Printer0) onto the workspace.

- Connect Cables: Connect each device to switch ports Fa0/1 through Fa0/6 using Copper Straight-Through Cable.

- Configure PCs: Open each PC → Click Desktop → IP Configuration → Choose Static → Enter IP (192.168.1.10 to 192.168.1.14) and Subnet Mask 255.255.255.0.

- Configure Printer: Open Printer0 → Click Config → FastEthernet0 → Enter IP 192.168.1.20, Subnet Mask 255.255.255.0, and ensure Port Status is checked ON.

- Wait for Link: Wait 30 seconds for amber switch lights to turn solid green.

3. Verification & Ping Output

Open PC0 → Desktop → Command Prompt:

Plaintext

PC> ping 192.168.1.11

Reply from 192.168.1.11: bytes=32 time<1ms TTL=128

PC> ping 192.168.1.20

Reply from 192.168.1.20: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.1.20:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

4. Conclusion

The LAN star topology was successfully built in Cisco Packet Tracer. All 5 PCs and the network printer received valid Class C IP addresses and ping replies confirmed 100% reachability without packet loss.

Task-4

Clean OS Installation

Ans:

Answer : I would perform the clean installation using a Linux ISO in VirtualBox or on the workstation.

1. I would create a new virtual machine or boot the workstation from the installation ISO.

2. I would select the required language, keyboard layout and installation option.

3. I would choose a clean/custom installation instead of an automatic upgrade.

4. I would manually create the required disk partitions. I would check the disk size before applying the changes so I do not select the wrong drive.

5. I would continue the OS installation and allow the system to restart.

6. After the first boot, I would install required drivers and run system updates.

7. I would check the startup applications and disable unnecessary programs that do not need to start with the OS.

8. I would check unnecessary services carefully and avoid disabling services required by the operating system or security software.

9. I would restart the computer and confirm that the OS boots without errors.

10. I would check the relevant Linux system tools to confirm that startup usage has improved.

Section-C

Mini Capstone Project

Mini Project: Nexus Innovations – Diagnosing Random System Reboots

Scenario : Three rendering workstations pass POST every morning but restart randomly during heavy afternoon workloads. The computers are kept inside poorly ventilated desk cabinets.

1. Three Hardware Components I Would Check First

If the systems pass POST but restart during heavy workloads, I would first check the following three hardware areas:

• 1. CPU/GPU cooling system – Rendering creates high CPU and GPU load. If airflow is poor, temperatures can rise until the system becomes unstable or protection mechanisms cause a shutdown/restart.

• 2. PSU – A rendering workload can increase power demand. If the PSU is overloaded, aging or unstable, the system may restart when the CPU and GPU draw more power.

• 3. RAM and motherboard power delivery – Unstable RAM or motherboard power delivery can cause crashes under sustained load. I would check RAM seating, memory errors and motherboard temperature/diagnostic information.

2. Why the Systems Pass POST in the Morning but Fail in the Afternoon

POST happens when the computer starts and checks the basic hardware. Passing POST does not mean the system will remain stable under a long heavy workload.

In the morning, the systems may be cool because they have not been under load. During rendering in the afternoon, the CPU and GPU generate much more heat. Because the workstations are inside poorly ventilated cabinets, hot air can build up and the cooling system may not remove heat effectively.

If temperatures become too high, the CPU or GPU can reduce performance through thermal throttling, and continued overheating can result in instability or a restart. A PSU problem may also appear only under higher load because power demand is much greater during rendering.

3. Stress Test to Confirm a Thermal or Power Issue

I would use a suitable CPU/GPU stress-testing tool and monitor temperatures, clock speeds, system stability and power-related symptoms.

1. Record the normal idle CPU and GPU temperatures before starting the test.

2. Run a controlled CPU stress test and monitor the temperature.

3. Run a GPU stress test separately and monitor the temperature.

4. If possible, run a combined CPU and GPU workload for a controlled period while watching temperatures and system behaviour.

5. Stop the test if temperatures reach an unsafe level or the system becomes unstable.

6. If the system restarts when temperatures rise sharply, that would support a thermal cause. If the restart happens mainly when both CPU and GPU power demand is high while temperatures remain reasonable, I would investigate the PSU and power delivery.

4. Three Specific Remediation Steps

1. Improve airflow – Move the workstations out of the enclosed cabinets or provide proper intake and exhaust airflow. I would also clean dust from fans and heatsinks.

2. Check and improve cooling – Verify that CPU/GPU fans are working, check the CPU cooler mounting and thermal paste, and confirm that case airflow is not blocked.

3. Check power and hardware stability – Test the PSU, inspect power connectors, test RAM, and monitor CPU/GPU temperatures during the workload. Replace a faulty or insufficient component if testing confirms the problem.

5. PSU Upgrade Justification

A PSU upgrade might be necessary if the existing PSU does not provide enough wattage or stable power for the CPU and GPU under full rendering load. I would first check the PSU's rated capacity, age, efficiency and required power connectors and compare this with the system's actual hardware requirements.

However, I would not replace the PSU only because the computers are restarting. If temperature testing shows that the CPU or GPU is overheating because of the poorly ventilated cabinets, improving cooling should be the first action. The PSU should be upgraded when testing or power calculations show that the current unit is insufficient or unstable.

Section-D

AI-Augmented Learning

Assignment: IP Addressing & Subnetting Plan — Nexus Innovations

Item 1: Exact Prompt(s) Used with the AI Tool

Prompt:

"Act as a network engineer. Create a Class C IP addressing and subnetting plan for Nexus Innovations' startup office network. The office has 5 workstations (Workstation-1 to Workstation-5) and 1 Network Printer. Use a subnet large enough for current needs plus growth, but do not waste a full /24. Provide: network address, subnet mask, usable host range, broadcast address, IP assignment table for all 6 devices, and specifically reserve addresses for future expansion."

Item 2: Comparison of AI's Original Plan and Corrected Version

A. Subnet Specification Comparison

| Parameter | AI Original Plan (/29) | Corrected Plan (/28) | Verification Remarks |
| --- | --- | --- | --- |
| Network Address | 192.168.10.0 /29 | 192.168.10.0 /28 | Expanded prefix length to /28 |
| Subnet Mask | 255.255.255.248 | 255.255.255.240 | Correct mask for 14 usable hosts |
| Usable Host Range | 192.168.10.1 – 192.168.10.6 | 192.168.10.1 – 192.168.10.14 | AI plan had zero growth capacity |
| Broadcast Address | 192.168.10.7 | 192.168.10.15 | Shifted to correct /28 boundary |
| Default Gateway | 192.168.10.1 | 192.168.10.1 | Preserved for router interface |

B. Device IP Allocation Table

| Device / Role | AI Original IP | Corrected IP | Subnet Mask | Status |
| --- | --- | --- | --- | --- |
| Default Gateway (Router) | 192.168.10.1 | 192.168.10.1 | 255.255.255.240 | Active |
| Workstation-1 | 192.168.10.2 | 192.168.10.2 | 255.255.255.240 | Active |
| Workstation-2 | 192.168.10.3 | 192.168.10.3 | 255.255.255.240 | Active |
| Workstation-3 | 192.168.10.4 | 192.168.10.4 | 255.255.255.240 | Active |
| Workstation-4 | 192.168.10.5 | 192.168.10.5 | 255.255.255.240 | Active |
| Workstation-5 | 192.168.10.6 | 192.168.10.6 | 255.255.255.240 | Active |
| Network Printer | 192.168.10.7 (Conflict) | 192.168.10.10 | 255.255.255.240 | Active (Static) |
| Future Workstations (Growth) | None (Exhausted) | 192.168.10.7 – 192.168.10.9 | 255.255.255.240 | Reserved |
| Future Infrastructure (Growth) | None (Exhausted) | 192.168.10.11 – 192.168.10.14 | 255.255.255.240 | Reserved |

Item 3: Explanatory Note (Changes Made & Why)

- The AI generated a /29 subnet (255.255.255.248), which yields only 6 usable host addresses (192.168.10.1–192.168.10.6), and it mistakenly assigned the subnet's broadcast address (192.168.10.7) to the network printer, causing a critical IP conflict and packet drops.

- I changed the subnet mask from /29 to /28 (255.255.255.240), increasing usable host capacity from 6 to 14 (192.168.10.1–192.168.10.14) with a valid broadcast address of 192.168.10.15.

- This resolved the printer's invalid IP conflict by reallocating it to 192.168.10.10 and provided 7 unallocated addresses (192.168.10.7–192.168.10.9 and 192.168.10.11–192.168.10.14) reserved for future expansion.